Quiz

Top of Form

Question 1 (1 point)

Saved

What feature does UDP provide as a Layer 4 protocol?

Question 1 options:

|  | Multiplexing via port numbering |
| --- | --- |
|  | Reliability |
|  | Flow control |
|  | Sequencing |

Question 2 (1 point)

Saved

What is the port number for Telnet?

Question 2 options:

|  | 21 |
| --- | --- |
|  | 23 |
|  | 53 |
|  | 80 |

Question 3 (1 point)

Saved

EUI-64 stands for.

Question 3 options:

|  | Empire United International-64 |
| --- | --- |
|  | Extended Unique Identifier-64 |
|  | Express Universal Interface |

Question 4 (1 point)

Saved

Which of the following IP addresses is considered a link-local IPv6 address?

Question 4 options:

|  | FE80::/10 |
| --- | --- |
|  | FECO::/10 |
|  | ::/128 |
|  | 200::/16 |

Question 5 (1 point)

Saved

Choose two types of multicasts IPv6 addresses.

Question 5 options:

|  | Well-known multicast address |
| --- | --- |
|  | Solicitation node multicast address |
|  | LLA multicast address |

Question 6 (1 point)

Saved

What is the purpose of the FTP application?

Question 6 options:

|  | Transfer email files from a client to a mail server using UDP |
| --- | --- |
|  | Transfer data files from one end device to another using UDP |
|  | Transfer data files from one end device to another using TCP |
|  | Transfer email files from a client to a mail server using TCP |

Question 7 (1 point)

Saved

Which of the following items is considered a Multicast address?

Question 7 options:

|  | ff00::/8. |
| --- | --- |
|  | Ee00::/9 |
|  | Dd01:/8 |
|  | Ffff:/8 |

Question 8 (1 point)

Saved

Which of the following is not a TCP/IP Application layer protocol?

Question 8 options:

|  | Telnet |
| --- | --- |
|  | SMTP |
|  | HTTP |
|  | ARP |

Question 9 (1 point)

Saved

IPv6 uses the dotted-decimal subnet mask notation.

Question 9 options:

|  | True |
| --- | --- |
|  | False |

Question 10 (1 point)

Saved

SLAAC allows a device to configure a GUA without the services of DHCPv6. no DHCP.

Question 10 options:

|  | True |
| --- | --- |
|  | False |

Question 11 (1 point)

Saved

The subnet ID field is the area between the ___________________.

Question 11 options:

|  | Multicast Prefix and the interface ID |
| --- | --- |
|  | IP v4 Prefix and the interface ID |
|  | Local link Prefix and the interface ID |
|  | Global Routing Prefix and the interface ID |

Question 12 (1 point)

Saved

Compress the following IPv6 address: 2001:0000:0000:0001:001B:1000:0000:0178

Question 12 options:

|  | 2001::1:1B:1:178 c. |
| --- | --- |
|  | 2001::1:1B:1000:0:178 |
|  | 2001::1:1B:0:178 |
|  | 2001::1:1B:1:0:178 |

Question 13 (1 point)

Saved

A network service provider assigns you a global routing prefix of 2001:db9:acad::/48. What is the compressed network address for subnet 38 decimal?

Question 13 options:

|  | 2001:db9:acad:0038::/64 |
| --- | --- |
|  | 2001:db9:acad:0038:0000:0000:0000:0000:/64 |
|  | 2001:db9:acad:0026::/64 |
|  | 2001:db9:acad:002f::/64 |

Question 14 (1 point)

Saved

Which of the following defines a network socket?

Question 14 options:

|  | A matching pair of ICMP echo request and reply Identifiers. |
| --- | --- |
|  | Something that you plug a hair dryer or a lamp into |
|  | 172.16.254.16:80 |
|  | A matching pair of SYN and ACK numbers |

Question 15 (1 point)

Saved

IEEE inserts a ______ bits value of _____ into ________ of the ________ address

Question 15 options:

|  | 16, aaaa, beginning, MAC |
| --- | --- |
|  | 16, fffe, middle, MAC |
|  | 16, cbfe, middle, IP |
|  | 16, ffcc, beginning, IP |

Question 16 (1 point)

Saved

Complete the following sentence: Sockets pairs use

Question 16 options:

|  | The source port number to act as a return address for the requesting application. |
| --- | --- |
|  | The destination port number to act as a return address for the requesting application. |
|  | The physical addresses of the devices uniquely identify processes on different |
|  | machines. |
|  | IP source and destination addresses to uniquely identify processes on different machines |

Question 17 (1 point)

Saved

How would a device obtain an IPv6 address from a router?

Question 17 options:

|  | Through ICMPv6 message |
| --- | --- |
|  | Through DNSv6 message |
|  | Through FTPv6 message |
|  | Through HTTPv6 message |

Question 18 (1 point)

Saved

Which of the following is NOT a valid method for obtaining an IPv6 address?

Question 18 options:

|  | SLAAC (RA) and DHCPv6 |
| --- | --- |
|  | NDP |
|  | DHCPv6 |
|  | SLAAC (RA) |

Question 19 (1 point)

Saved

Which of the following protocols handles all tasks associated with dividing the data stream into segments, providing reliability, controlling data flow, and reordering segments?

Question 19 options:

|  | DHCP |
| --- | --- |
|  | DNS |
|  | TCP |
|  | FTP |

Question 20 (1 point)

Saved

Which of the following items uses UDP protocol? (Select all that apply)

Question 20 options:

|  | Handshake |
| --- | --- |
|  | Video conferencing |
|  | VoIP |
|  | TFTP |

Question 21 (1 point)

Saved

which of the following tools help administrators to migrate from IPv4 to IPv6?

Question 21 options:

|  | Dual stack |
| --- | --- |
|  | Tunneling |
|  | Transition |
|  | Multiplexing |

Question 22 (1 point)

Saved

In the EUI-64 method to obtain an IPv6 address the ____th bit of the client MAC address is reversed from binary 0 to 1.

Question 22 options:

|  | 4 |
| --- | --- |
|  | 5 |
|  | 6 |
|  | 7 |

Question 23 (1 point)

Saved

The dual-stack migration techniques is a way to support IPv4 and IPv6 by:

Question 23 options:

|  | Translating IPv6 packets into IPv4, and vice versa. |
| --- | --- |
|  | Having devices run both IPv4 and IPv6 protocol stacks simultaneously. |
|  | Encapsulating IPv6 packets in IPv4. |
|  | Doubling the height of the TCP/IP stack, by placing IPv4 on top of IPv6. |

Question 24 (1 point)

Saved

What are the three methods RA uses to configure a GUA IPv6?

Question 24 options:

|  | Stateless Address Auto-configuration (SLAAC) |
| --- | --- |
|  | SLAAC with stateless DHCPv6 server |
|  | Stateful DHCPv6 (no SLAAC) |
|  | Switch IP addresses |

Question 25 (1 point)

Saved

According to the lecture, the second rule to compress an IPv6 address is:

Question 25 options:

|  | Omit all trailer zero. |
| --- | --- |
|  | Omit all consecutive zeros and replace them with two colons. |
|  | Omit all consecutive zeros and replace them with one colon. |
|  | Omit all leading zeros. |

Question 26 (1 point)

Saved

What is a router solicitation message?

Question 26 options:

|  | It is a message from a device, to discover a router |
| --- | --- |
|  | It is a message from a device to discover a switch |
|  | It is a message from a device to discover another PC |
|  | It is a multicast message from a device to discover a Hub. |

Question 27 (1 point)

Saved

Through SLAAC, the prefix is provided by the RA and the device uses either the EUI-64 or random generation method to create an interface ID.

Question 27 options:

|  | True |
| --- | --- |
|  | False |

Question 28 (1 point)

Saved

URL is an acronym for?

Question 28 options:

|  | Unified Restore Locator |
| --- | --- |
|  | Uniform Restore Locator |
|  | Uniform Resource Locator |
|  | Unified Resource Locator |

Question 29 (1 point)

Saved

How will hosts find the DHCP server?

Question 29 options:

|  | The DHCP server address must be configured |
| --- | --- |
|  | It is always the 1st usable IP address |
|  | They will use a layer 2 broadcast |
|  | They will use the default gateway |

Question 30 (1 point)

Saved

TCP is a _______protocol which means it keeps track of the state of the communication session.

Question 30 options:

|  | Stateful |
| --- | --- |
|  | Careless |
|  | Connectionless |
|  | Best effort |

Question 31 (1 point)

Saved

To ensure the uniqueness of any IPv6 unicast address, the client may use a process known as Duplicate Address Detection (DAD).

Question 31 options:

|  | True |
| --- | --- |
|  | False |

Question 32 (1 point)

Saved

According to the lecture, the first rule to shorten an IPv6 is to

Question 32 options:

|  | Omit the leading zeros |
| --- | --- |
|  | Omit the trailer zeros |
|  | Omit the middle zero |
|  | Omit leading ones |

Question 33 (1 point)

Saved

What is a Router Advertisement message?

Question 33 options:

|  | It is a message that comes from a switch to inform hosts on how to obtain an IPv6 GUA |
| --- | --- |
|  | It is a message that comes from another PC to inform hosts on how to obtain an IPv6 GUA |
|  | It is a message that comes from the router to inform hosts on how to obtain an IPv6 GUA |
|  | It is a message that does all the above |

Question 34 (1 point)

Saved

Which service does the DNS application provide?

Question 34 options:

|  | It translates the alphanumeric internet address (URL) into the destination IP address and |
| --- | --- |
|  | TCP port number |
|  | It translates the alphanumeric internet address (URL) into the next hop IP address |
|  | It translates the alphanumeric internet address (URL) into the source IP address |
|  | It translates the alphanumeric internet address (URL) into the destination IP address |

Question 35 (1 point)

Saved

Which of the following protocols does: Data is reconstructed in the order that it is received, Any segments that are lost are not resent, There is no session establishment, The sending is not informed about resource availability

Question 35 options:

|  | TCP |
| --- | --- |
|  | UDP |
|  | FTP |
|  | HTTP |

Submit Quiz35 of 35 questions saved

Bottom of Form