Quiz

Top of Form

Question 1 (1 point)

Saved

Which layer of the OSI model deals with bits cross?

Question 1 options:

|  | Physical |
| --- | --- |
|  | Network |
|  | Transport |
|  | Application |

Question 2 (1 point)

Saved

The first part of the MAC address is considered ______

Question 2 options:

|  | Organization Unique Identifier |
| --- | --- |
|  | Old Unit Interface |
|  | Organic University interconnect |
|  | Option Unit Interface |

Question 3 (1 point)

Saved

Carrier sense multiple access with collision avoidance (CSMA/CA) is used on Wireless LAN.

Question 3 options:

|  | True |
| --- | --- |
|  | False |

Question 4 (1 point)

Saved

Which of the following cables are immune to EMI?

Question 4 options:

|  | Fiber optic |
| --- | --- |
|  | UTP |
|  | STT |
|  | Coaxial |

Question 5 (1 point)

Saved

The port number in a PC is the physical number

Question 5 options:

|  | True |
| --- | --- |
|  | False |

Question 6 (1 point)

Saved

The process that a source host uses to determine the destination MAC address associated with an IPv4 address is known as

Question 6 options:

|  | Tracer |
| --- | --- |
|  | ipconfig |
|  | Arp |
|  | ping |

Question 7 (1 point)

Saved

Layer 2 addressing is used only for remote delivery.

Question 7 options:

|  | True |
| --- | --- |
|  | False |

Question 8 (1 point)

Saved

What is the command to create a static MAC address?

Question 8 options:

|  | Arp -a |
| --- | --- |
|  | Arp -s |
|  | Arp -p |
|  | Arp -d |

Question 9 (1 point)

Saved

Which of the OSI model layers adds a Trailer (CRC) to the signal?

Question 9 options:

|  | Application |
| --- | --- |
|  | Physical |
|  | Network |
|  | Data link |

Question 10 (1 point)

Saved

According to the video (we watched) Which of the following numbers is considered a well-known port number?

Question 10 options:

|  | 0 to 1023 |
| --- | --- |
|  | 1024-49151 |
|  | 49152-65535 |

Question 11 (1 point)

Saved

In lab four, which logical port did you use to connect to the server

Question 11 options:

|  | 9010 |
| --- | --- |
|  | 8088 |
|  | 1024 |
|  | 443 |

Question 12 (1 point)

Saved

What PDU is received by the physical layer for encoding and transmission?

Question 12 options:

|  | Frame |
| --- | --- |
|  | Segment |
|  | Packet |
|  | Bit |

Question 13 (1 point)

Saved

What is the destination MAC address if it is a broadcast frame?

Question 13 options:

|  | 192.168.1.6 |
| --- | --- |
|  | FF-FF-FF-FF-FF-FF |
|  | 1-2-3-4-5-6 |
|  | Ethernet doesn't have a broadcast frame. |

Question 14 (1 point)

Saved

When the destination IP address is on a remote network, the destination MAC address is that of the ________

Question 14 options:

|  | PC2 MAC address |
| --- | --- |
|  | PC2 IP address |
|  | R2 MAC address |
|  | R1 MAC address |

Question 15 (1 point)

Saved

In lab 3, what was the port number for a secure HTTP?

Question 15 options:

|  | 423 |
| --- | --- |
|  | 413 |
|  | 443 |
|  | 433 |

Question 16 (1 point)

Saved

Full-duplex communications with Ethernet switches do not require access control through CSMA/CD.

Question 16 options:

|  | True |
| --- | --- |
|  | False |

Question 17 (1 point)

Saved

A device uses ARP to determine the ________ address of a local device when it knows its IPv4 address.

Question 17 options:

|  | Source IP address |
| --- | --- |
|  | Destination MAC address |
|  | Source MAC address |
|  | Destination IP address |

Question 18 (1 point)

Saved

If the source MAC address does not exist in the MAC address table, then what's going to happen?

Question 18 options:

|  | It is added to the table along with the incoming port number. |
| --- | --- |
|  | it is deleted in the table along with the incoming port number. |
|  | it is added to the router table along with the incoming port number. |
|  | it is dropping the frame, along with the incoming port number. |

Question 19 (1 point)

Saved

The limitations of wireless are: (select all that apply)

Question 19 options:

|  | Coverage area |
| --- | --- |
|  | Interference |
|  | Security |
|  | Shared medium |

Question 20 (1 point)

Saved

To send a frame, a device will search its ARP table for a destination IPv4 address and a corresponding MAC address. Now, If there is no ARP table entry is found in the PC, then the device sends an

Question 20 options:

|  | A unicast frame would be sent to the device directly |
| --- | --- |
|  | A broadcast request frame would be sent to all devices |
|  | A multicast request frame would be sent to all devices |
|  | A multicast request frame would be sent to the device directly |

Question 21 (1 point)

Saved

According to this image, which of the following numbers are considered the Organization Unique Identifier (vendor specific)?

Question 21 options:

|  | A1:B4:C5 |
| --- | --- |
|  | C1:DD:3E |
|  | DD:3E:A1 |
|  | B4:C5:C1 |

Question 22 (1 point)

Saved

Wireless use ____________ standard

Question 22 options:

|  | Wi-Fi (IEEE 802.1) |
| --- | --- |
|  | Wi-Fi (IEEE 802.2) |
|  | Wi-Fi (IEEE 802.19) |
|  | Wi-Fi (IEEE 802.11) |

Question 23 (1 point)

Saved

The Data Link Address is used to forward the frame from one device to another within.

Question 23 options:

|  | Different network segments |
| --- | --- |
|  | The Internet |
|  | The same device (inside the PC) |
|  | The same network segment |

Question 24 (1 point)

Saved

A MAC address is _____ number

Question 24 options:

|  | 48 bits |
| --- | --- |
|  | 48 bytes |
|  | 48 kilobytes |
|  | 48 megabytes |

Question 25 (1 point)

Saved

Local hosts in a network have the same network portion of IP address as the default gateway.

Question 25 options:

|  | True |
| --- | --- |
|  | False |

Question 26 (1 point)

Saved

Name three types of copper cabling.

Question 26 options:

|  | UTP |
| --- | --- |
|  | STP |
|  | Coaxial |
|  | Fiber optic |

Question 27 (1 point)

Saved

The amount of time, including delays, for data to travel from one given point to another is called

Question 27 options:

|  | Latency |
| --- | --- |
|  | Goodput |
|  | Throughput |

Question 28 (1 point)

Saved

According to the video (we watched in class) what was the port number for FTP

Question 28 options:

|  | 80 |
| --- | --- |
|  | 21 |
|  | 53 |
|  | 433 |

Question 29 (1 point)

Saved

Layer 2 addresses are used to deliver packets from one NIC to another NIC on the same network.

Question 29 options:

|  | True |
| --- | --- |
|  | False |

Question 30 (1 point)

Saved

Which sublayer of the Data link is responsible for encapsulation and media access control?

Question 30 options:

|  | Logical link control |
| --- | --- |
|  | Media access control |
|  | Application |
|  | transport |

Question 31 (1 point)

Saved

Which of the following commands shows the port number?

Question 31 options:

|  | Arp -a |
| --- | --- |
|  | Netstat -n |
|  | ipconfig /all |
|  | Cmd |

Question 32 (1 point)

Saved

When a NIC receives an Ethernet frame, it examines the ________ address to see if it matches the physical MAC address that is stored in RAM.

Question 32 options:

|  | Source IP address |
| --- | --- |
|  | Source MAC address |
|  | Destination MAC address |
|  | Destination IP address |

Question 33 (1 point)

Saved

Which of the following commands shows the ARP cache in your laptop?

Question 33 options:

|  | Arp -d |
| --- | --- |
|  | Arp -a |
|  | Arp -s |
|  | Arp -p |

Question 34 (1 point)

Saved

Which media uses patterns of microwaves to represent bits?

Question 34 options:

|  | copper |
| --- | --- |
|  | Wireless |
|  | fiber-optic |
|  | RJ45 |

Question 35 (1 point)

Saved

What type of information is contained in an ARP table?

Question 35 options:

|  | Routes to reach destination networks |
| --- | --- |
|  | Domain name to IPv4 address mapping |
|  | IPv4 address to MAC address mappings |
|  | Switch ports associated with destination MAC address |

Question 36 (1 point)

Saved

In lab 4, when you connected two routers, what type of cable did you use? (WAN to WAN)

Question 36 options:

|  | Straight thru cable |
| --- | --- |
|  | Crossover cable |
|  | Fiber optic cable |
|  | Coaxial cable |

Question 37 (1 point)

Saved

CSMA/CD works on which layer of the OSI model?

Question 37 options:

|  | Application |
| --- | --- |
|  | Network |
|  | Data link |
|  | Physical |

Question 38 (1 point)

Saved

When the destination IP address is on a remote network, the destination MAC address will be the address of the host's ________

Question 38 options:

|  | Host's default gateway, the router's NIC |
| --- | --- |
|  | Local default gateway, of the host's NIC |
|  | Host's default gateway, the switch's NIC |
|  | The host's IP address, the Hub's NIC |

Question 39 (1 point)

Saved

UTP wires are twisted to prevent the _________

Question 39 options:

|  | Attenuation |
| --- | --- |
|  | Crosstalk |
|  | Breakage cable |
|  | None of the above |

Question 40 (1 point)

Saved

In lab 4, what command did you use to obtain MAC addresses on your laptop?

Question 40 options:

|  | Arp -a |
| --- | --- |
|  | Arp -b |
|  | Arp -c |
|  | Arp -d |

Question 41 (1 point)

Saved

UTP cable has ____ pairs of wires.

Question 41 options:

|  | 4 |
| --- | --- |
|  | 6 |
|  | 8 |
|  | 10 |

Question 42 (1 point)

Saved

If PC1 wants to communicate with PC2, what is the destination MAC address of the frame when it comes out of the R1.

Question 42 options:

|  | bb-bb-bb |
| --- | --- |
|  | dd-dd-dd |
|  | ee-ee-ee |
|  | 55-55-55 |

Question 43 (1 point)

Saved

Which of the following commands provides this image?

Question 43 options:

|  | Arp -a |
| --- | --- |
|  | Ping -s |
|  | ipconfig /all |
|  | Tracert |

Question 44 (1 point)

Saved

Legacy Ethernet using a bus topology or hubs, is a shared, half-duplex medium

Question 44 options:

|  | True |
| --- | --- |
|  | False |

Question 45 (1 point)

Saved

The data link layer has two sublayers.

Question 45 options:

|  | Logical link control |
| --- | --- |
|  | Media access control |
|  | Application |
|  | Transport |

Question 46 (1 point)

Saved

What destination MAC address would be included in a frame sent from a source device to a destination device on the same local network?

Question 46 options:

|  | A broadcast MAC address of FF-FF-FF-FF-FF-FF |
| --- | --- |
|  | The MAC address of the destination device |
|  | The MAC address of the local router interface |

Question 47 (1 point)

Saved

Why do devices on the Ethernet network have to use two addresses (IP and MAC)? (select all that apply)

Question 47 options:

|  | Because MAC addresses never change when you move your PC to another network |
| --- | --- |
|  | Because IP addresses change when you move your PC to another network |
|  | Because both MAC and IP addresses change when you move your PC to another network |
|  | Because none of the MAC and IP addresses change when you move your PC to another network |

Question 48 (1 point)

Saved

At each hop along the path, a router performs __ basic Layer 2 functions: (select all that apply)

Question 48 options:

|  | Accepts a frame from the network medium. |
| --- | --- |
|  | De-encapsulates the frame to expose the encapsulated packet. |
|  | Re-encapsulates the packet into a new frame. |
|  | Forwards the new frame on the medium of the next network segment. |

Question 49 (1 point)

Saved

What two protocols are used to determine the MAC address of a known destination device IP address (IPv4 and IPv6) (choose two)

Question 49 options:

|  | DHCP |
| --- | --- |
|  | ARP |
|  | DNS |
|  | ND |

Question 50 (1 point)

Saved

According to the video, which of the following port numbers deals with email servers?

Question 50 options:

|  | 21 |
| --- | --- |
|  | 80 |
|  | 53 |
|  | 25 |

Submit Quiz50 of 50 questions saved

Bottom of Form