Quiz

Top of Form

Question 1 (1 point)

Saved

Where does this MAC address dd-45-23-a6-ba-aa work in the OSI model?

Question 1 options:

|  | Transport |
| --- | --- |
|  | Network |
|  | Data link |
|  | Physical |

Question 2 (1 point)

Saved

Which of the following commands will show the ARP cache?

Question 2 options:

|  | ARP -a |
| --- | --- |
|  | ARP -b |
|  | ARP -s |
|  | ARP -d |

Question 3 (1 point)

Saved

Which of the following items is used to differentiate the network portion from the host portion of an IPv4 address?

Question 3 options:

|  | IP address |
| --- | --- |
|  | Subnet mask |
|  | Default gateway |
|  | Host IP address |

Question 4 (1 point)

Saved

Which of the following protocols does NOT synchronization and Acknowledgement?

Question 4 options:

|  | DNS |
| --- | --- |
|  | DHCP |
|  | UDP |
|  | TCP |

Question 5 (1 point)

Saved

Every frame that enters a switch is checked for new information to learn. It does this by examining the _____________address of the frame and the port number where the frame entered the switch

Question 5 options:

|  | Source MAC addresses |
| --- | --- |
|  | Destination MAC address |
|  | Source IP address |
|  | Destination IP address |

Question 6 (1 point)

Saved

Which of the IPv4 addresses are not routable?

Question 6 options:

|  | Private IP address |
| --- | --- |
|  | Public IP address |
|  | Private MAC address |
|  | Public MAC address |

Question 7 (1 point)

Saved

Which field in the IPv6 packet is used by the router to determine if a packet has expired and should be dropped?

Question 7 options:

|  | No route to the destination |
| --- | --- |
|  | Address unreachable. |
|  | TTL |
|  | Hop limit |

Question 8 (1 point)

Saved

Which of the following devices is considered a half duplex?

Question 8 options:

|  | Switch |
| --- | --- |
|  | Hub |
|  | Router |
|  | All the above |

Question 9 (1 point)

Saved

The functions of Dynamic routing are(router). (Choose three)

Question 9 options:

|  | Discover remote networks |
| --- | --- |
|  | Maintain up-to-date information |
|  | Choose the best path to the destination |
|  | Ignore the packet. |

Question 10 (1 point)

Saved

Which of the following protocols would exchange three handshakes?

Question 10 options:

|  | UDP |
| --- | --- |
|  | TCP |
|  | DNS |
|  | DHCP |

Question 11 (1 point)

Saved

Which of the IPv4 addresses are not routable?

Question 11 options:

|  | Private IP address |
| --- | --- |
|  | Public IP address |
|  | Private MAC address |
|  | Public MAC address |

Question 12 (1 point)

Saved

Where does the IPv6 address work in the OSI model?

Question 12 options:

|  | Transport |
| --- | --- |
|  | Network |
|  | Data link |
|  | Physical |

Question 13 (1 point)

Saved

Which command is used in Windows to find the hops number?

Question 13 options:

|  | ipconfig /all |
| --- | --- |
|  | Tracert |
|  | Trace |
|  | Ping |

Question 14 (1 point)

Saved

Switches propagate broadcasts out all interfaces except the interface on which it was received.

Question 14 options:

|  | True |
| --- | --- |
|  | False |

Question 15 (1 point)

Saved

According to the lecture, the minimum size for the Ethernet frame is ____ and the maximum is ______.

Question 15 options:

|  | 64 and 1500 bytes |
| --- | --- |
|  | 64 and 1500 kilobytes |
|  | 64 and 1500 megabytes |
|  | 64 and 1500 gigabytes |

Question 16 (1 point)

Saved

Refer to the exhibit. According to the lecture, which two fields are very important in the header packet? (Choose two).

Question 16 options:

|  | Source IP address |
| --- | --- |
|  | Source MAC address |
|  | Destination IP address |
|  | Destination MAC address |

Question 17 (1 point)

Saved

Where IP address 192.168.1.5 work in the OSI model?

Question 17 options:

|  | Transport |
| --- | --- |
|  | Network |
|  | Data link |
|  | Physical |

Question 18 (1 point)

Saved

In IPv4, how does a device determine whether that destination is local or remote?

Question 18 options:

|  | By checking local IPv4 address |
| --- | --- |
|  | By checking the local IPv6 address |
|  | By comparing the subnet mask and the IP address (ANDing) |
|  | By sending the packet to the switch's interface |

Question 19 (1 point)

Saved

According to the lecture, which of the two following protocols works in the network layer? (Choose two)

Question 19 options:

|  | ICMP |
| --- | --- |
|  | IP |
|  | DNS |
|  | DHCP |

Question 20 (1 point)

Saved

IPv6 address is ____bits.

Question 20 options:

|  | 32 |
| --- | --- |
|  | 64 |
|  | 128 |
|  | 255 |

Question 21 (1 point)

Saved

An Ethernet MAC address is a physical address burned on a NIC card. The MAC address number is represented in Hexadecimal numbers. Which part of this image is considered the OUI (Organization Unit Identifier).

Question 21 options:

|  | The first 3 bytes |
| --- | --- |
|  | The last three bytes |
|  | The three middle bytes |
|  | The whole 6 bytes |

Question 22 (1 point)

Saved

Which of the following defines a network socket?

Question 22 options:

|  | A matching pair of ICMP echo requests and reply to Identifiers. |
| --- | --- |
|  | something that you plug a hair dryer or a lamp into |
|  | 172.16.254.16:80 |
|  | A matching pair of SYN and ACK numbers |

Question 23 (1 point)

Saved

Choose three forwarding methods that the switch uses

Question 23 options:

|  | Store and forwarding |
| --- | --- |
|  | Cut through |
|  | Fragment free |
|  | Jumbo forwarding |

Question 24 (1 point)

Saved

Which of the following protocols work in the Application layer (choose three)

Question 24 options:

|  | UDP |
| --- | --- |
|  | DHCP |
|  | HTTP |
|  | SNMP |

Question 25 (1 point)

Saved

There is a network with DHCP on. For some reason, the DHCP server on the Windows network goes down. Which IP address these Windows clients use to continue communication?

Question 25 options:

|  | 192.168.1.0 |
| --- | --- |
|  | 169.254.1.5 |
|  | 127.0.0.1 |
|  | Clients can not communicate without a DHCP server. |

Question 26 (1 point)

Saved

When you turn on your computer on a network, it looks for a DHCP server on the network to have an IP address. How does your PC locate the DHCP server on the network?

Question 26 options:

|  | Unicast |
| --- | --- |
|  | Multicast |
|  | Broadcast |
|  | anycast |

Question 27 (1 point)

Saved

What are the characteristics of hosts on different network segments?

Question 27 options:

|  | They share the same network ID. |
| --- | --- |
|  | They share the same subnet mask. |
|  | They have different default gateway addresses |
|  | They share the same broadcast address. |

Question 28 (1 point)

Saved

Which of the following items would change the public IP address to a private IP address?

Question 28 options:

|  | Switch table |
| --- | --- |
|  | Router table |
|  | NAT |
|  | ARP -A |

Question 29 (1 point)

Saved

The network layer performs the following basic operations. (Select all that apply)

Question 29 options:

|  | Addressing end devices |
| --- | --- |
|  | Encapsulation |
|  | Routing |
|  | De-encapsulation |

Question 30 (1 point)

Saved

What will a host on an Ethernet network do if it receives a frame with a destination MAC address that does not match its own MAC address?

Question 30 options:

|  | It will remove the frame from the media. |
| --- | --- |
|  | It will strip off the data-link frame to check the destination IP address. |
|  | It will forward the frame to the next host. |
|  | It will discard the frame |

Question 31 (1 point)

Saved

If an ethernet frame is less than 64 bytes, it is called

Question 31 options:

|  | Runs |
| --- | --- |
|  | Runt |
|  | Bunt |
|  | Dunt |

Question 32 (1 point)

Saved

Which of the following protocols uses the best effort delivery?

Question 32 options:

|  | DNS |
| --- | --- |
|  | FTP |
|  | UDP |
|  | TCP |

Question 33 (1 point)

Saved

Choose two sub-layer of data link in the OSI model

Question 33 options:

|  | LLC |
| --- | --- |
|  | MAC |
|  | LLD |
|  | IPD |

Question 34 (1 point)

Saved

The TTL field in the IP packet has 128 numbers. What does it mean, when the TTL field reaches zero?

Question 34 options:

|  | It means resending the packet again. |
| --- | --- |
|  | It means dropping the packet |
|  | It means changing the path. |
|  | It means guaranteed packet delivery. |

Question 35 (1 point)

Saved

What does MTU stand for?

Question 35 options:

|  | Maximum twist unit |
| --- | --- |
|  | More twisted Unit |
|  | Maximum Translate Universal |
|  | Maximum Transmission Unit |

Submit Quiz35 of 35 questions saved

Bottom of Form