- Network technologies are helping to create a world in which national borders, geographic distances, and physical limitations become less relevant, presenting ever-diminishing obstacles.

- Global communities allow for social interaction that is independent of location or time zone.

- All computers that are connected to a network and participate directly in network communication are classified as hosts.

- Hosts can be called end devices.

- Some hosts are also called clients.

- However, the term hosts specifically refers to devices on the network that are assigned a number for communication purposes.

- This number identifies the host within a particular network. This number is called the Internet Protocol (IP) address. An IP address identifies the host and the network to which the host is attached.

- Servers are computers with software that allow them to provide information, like email or web pages, to other end devices on the network. Each service requires separate server software. For example, a server requires web server software in order to provide web services to the network. A computer with server software can provide services simultaneously to many different clients.

- clients are a type of host. Clients have software for requesting and displaying the information obtained from the server.

- An example of client software is a web browser, like Chrome or FireFox. A single computer can also run multiple types of client software.

- For example, a user can check email and view a web page while instant messaging and listening to an audio stream. The table lists three common types of server software.

# 1.2.2 Peer-to-Peer

- Client and server software usually run on separate computers, but it is also possible for one computer to be used for both roles at the same time. In small businesses and homes, many computers function as the servers and clients on the network. This type of network is called a peer-to-peer network.

- the print sharing PC has a Universal Serial Bus (USB) connection to the printer and a network connection, using a network interface card (NIC), to the file sharing PC.

- To distinguish one end device from another, each end device on a network has an address.

- An end device is either the source or destination of a message transmitted over the network.

# 1.2.4 Intermediary Devices

- Intermediary devices connect the individual end devices to the network. They can connect multiple individual networks to form an internetwork.

- intermediary devices provide connectivity and ensure that data flows across the network.

- An Ethernet hub is also known as a multiport repeater. Repeaters regenerate and retransmit communication signals. Notice that all intermediary devices perform the function of a repeater.

# 1.2.5 Network Media

- Communication transmits across a network on media. The media provides the channel over which the message travels from source to destination.

- Metal wires within cables - Data is encoded into electrical impulses.

- Glass or plastic fibers within cables (fiber-optic cable) - Data is encoded into pulses of light.

- Wireless transmission - Data is encoded via modulation of specific frequencies of electromagnetic waves.

# 1.3.1 Network Representations

- A diagram provides an easy way to understand how devices connect in a large network. This type of “picture” of a network is known as a topology diagram.

- Network Interface Card (NIC) - A NIC physically connects the end device to the network.

- Physical Port - A connector or outlet on a networking device where the media connects to an end device or another networking device.

- Interface - Specialized ports on a networking device that connect to individual networks. Because routers connect networks, the ports on a router are referred to as network interfaces.

Note: The terms port and interface are often used interchangeably.

# 1.3.2 Topology Diagrams*

- Topology diagrams provide a visual map of how the network is connected. There are two types of topology diagrams: physical and logical.

1. A NIC is a specialized port on a networking device that connects to individual networks.
2. An interface physically connects the end device to the network.
3. The logical topology lets you see which end devices are connected to which intermediary devices and what media is being used.
4. The physical topology lets you see the actual location of intermediary devices and cable installation.

# 1.4.1 Networks of Many Sizes

- Small office and home office (SOHO) networks allow people to work from home, or a remote office.

- Businesses and large organizations use networks to provide consolidation, storage, and access to information on network servers.

- the term internet means a “network of networks”. It is a collection of interconnected private and public networks.

- In small businesses and homes, many computers function as both the servers and clients on the network. This type of network is called a peer-to-peer network.

- The SOHO network allows computers in a home office or a remote office to connect to a corporate network, or access centralized, shared resources.

# 1.4.2 LANs and WANs

Network infrastructures vary greatly in terms of:

- Size of the area covered

- Number of users connected

- Number and types of services available

- Area of responsibility

- A network serving a home, small building, or a small campus is considered a LAN.

- WANs are typically managed by service providers (SPs) or Internet Service Providers (ISPs).

- WANs typically provide slower speed links between LANs.

# 1.4.3 The Internet

- the internet as a collection of interconnected LANs and WANs.

- LANs use WAN services to interconnect.

- 1. A LAN provides access to users and end devices in a small geographical area.
2. An extranet provides secure and safe access to individuals who work for a different organization but require access to the organization’s data.
3. A WAN provides access to other networks over a large geographical area.

- Dial-up Telephone - An inexpensive option that uses any phone line and a modem. The low bandwidth provided by a dial-up modem connection is not sufficient for large data transfer, although it is useful for mobile access while traveling.

- The term network architecture, in this context, refers to the technologies that support the infrastructure and the programmed services and rules, or protocols, that move data across the network.

four basic characteristics that network architects must address to meet user expectations:

- Fault Tolerance

- Scalability

- Quality of Service (QoS)

- Security

# 1.6.2 Fault Tolerance

- A fault tolerant network is one that limits the number of affected devices during a failure. It is built to allow quick recovery when such a failure occurs. These networks depend on multiple paths between the source and destination of a message. If one path fails, the messages are instantly sent over a different link. Having multiple paths to a destination is known as redundancy.

- Implementing a packet-switched network is one way that reliable networks provide redundancy.

- Packet switching splits traffic into packets that are routed over a shared network.

- A single message, such as an email or a video stream, is broken into multiple message blocks, called packets.

- Each packet has the necessary addressing information of the source and destination of the message.

- The routers within the network switch the packets based on the condition of the network at that moment.

- This means that all the packets in a single message could take very different paths to the same destination.

- In the figure, the user is unaware and unaffected by the router that is dynamically changing the route when a link fails.

# 1.6.3 Scalability

- A scalable network expands quickly to support new users and applications. It does this without degrading the performance of services that are being accessed by existing users.

- Internet

- Additional users and whole networks can be connected to the internet without degrading performance for existing users.

# 1.6.4 Quality of Service

- Congestion occurs when the demand for bandwidth exceeds the amount available. Network bandwidth is measured in the number of bits that can be transmitted in a single second, or bits per second (bps). When simultaneous communications are attempted across the network, the demand for network bandwidth can exceed its availability, creating network congestion.

- When the volume of traffic is greater than what can be transported across the network, devices will hold the packets in memory until resources become available to transmit them.

- With a QoS policy in place, the router can manage the flow of data and voice traffic, giving priority to voice communications if the network experiences congestion.The focus of QoS is to prioritize time-sensitive traffic. The type of traffic, not the content of the traffic, is what is important.

# 1.6.5 Network Security

- two types of network security concerns: network infrastructure security and information security.

- Securing the network infrastructure includes physically securing devices that provide network connectivity and preventing unauthorized access to the management software that resides on them, as shown in the figure.

there are three primary requirements.

- Confidentiality - Data confidentiality means that only the intended and authorized recipients can access and read data.

- Integrity - Data integrity assures users that the information has not been altered in transmission, from origin to destination.

- Availability - Data availability assures users of timely and reliable access to data services for authorized users.

- There are several networking trends that affect organizations and consumers:

- Bring Your Own Device (BYOD)

- Online collaboration

- Video communications

- Cloud Computing

- Collaboration is defined as “the act of working with another or others on a joint project.”

- Cisco Webex Teams is a multifunctional collaboration tool that lets you send instant messages to one or more people, post images, and post videos and links. Each team ‘space’ maintains a history of everything that is posted there.

- Cloud computing is possible because of data centers.

- For security, reliability, and fault tolerance, cloud providers often store data in distributed data centers. Instead of storing all the data of a person or an organization in one data center, it is stored in multiple data centers in different locations.

- There are four primary types of clouds: Public clouds, Private clouds, Hybrid clouds, and Community clouds, as shown in the table.

- Powerline networking for home networks uses existing electrical wiring to connect devices

- Using a standard powerline adapter, devices can connect to the LAN wherever there is an electrical outlet. No data cables need to be installed, and there is little to no additional electricity used. Using the same wiring that delivers electricity, powerline networking sends information by sending data on certain frequencies.

- Powerline networking is especially useful when wireless access points cannot reach all the devices in the home. Powerline networking is not a substitute for dedicated cabling in data networks. However, it is an alternative when data network cables or wireless communications are not possible or effective.

You have successfully identified the correct answers.

1. Video communications is a good conferencing tool to use with others who are located elsewhere in your city, or even in another country.
2. BYOD feature describes using personal tools to access information and communicate across a business or campus network.
3. Cloud computing contains options such as Public, Private, Custom and Hybrid.
4. Powerline is being used when connecting a device to the network using an electrical outlet.
5. Wireless broadband uses the same cellular technology as a smartphone.

# 1.8.1 Security Threats

- Network security is an integral part of computer networking, regardless of whether the network is in a home with a single connection to the internet or is a corporation with thousands of users.

- Viruses, worms, and Trojan horses - These contain malicious software or code running on a user device.

- Spyware and adware - These are types of software which are installed on a user’s device. The software then secretly collects information about the user.

- Zero-day attacks - Also called zero-hour attacks, these occur on the first day that a vulnerability becomes known.

- Threat actor attacks - A malicious person attacks user devices or network resources.

- Denial of service attacks - These attacks slow or crash applications and processes on a network device.

- Data interception and theft - This attack captures private information from an organization’s network.

- Identity theft - This attack steals the login credentials of a user in order to access private data.

# 1.8.2 Security Solutions

- No single solution can protect the network from the variety of threats that exist. For this reason, security should be implemented in multiple layers, using more than one security solution. If one security component fails to identify and protect the network, others may succeed.

- Antivirus and antispyware - These applications help to protect end devices from becoming infected with malicious software.

- Firewall filtering - Firewall filtering blocks unauthorized access into and out of the network. This may include a host-based firewall system that prevents unauthorized access to the end device, or a basic filtering service on the home router to prevent unauthorized access from the outside world into the network.

- Dedicated firewall systems - These provide more advanced firewall capabilities that can filter large amounts of traffic with more granularity.

- Access control lists (ACL) - These further filter access and traffic forwarding based on IP addresses and applications.

- Intrusion prevention systems (IPS) - These identify fast-spreading threats, such as zero-day or zero-hour attacks.

- Virtual private networks (VPN) - These provide secure access into an organization for remote workers.

  - A DoS attack slows down or crashes equipment and programs.
2. A VPN creates a secure connection for remote workers.
3. A firewall blocks unauthorized access to your network.
4. A zero-day or zero-hour attack occurs on the first day that a vulnerability becomes known.
5. A virus, worm, or Trojan horse is malicious code running on user devices.