- Without the physical layer, you would not have a network.

# 4.1.1 The Physical Connection

- Whether connecting to a local printer in the home or a website in another country, before any network communications can occur, a physical connection to a local network must be established.

- With wireless devices, data is transmitted using radio waves.

- Devices on a wireless network must be connected to a wireless access point (AP) or wireless router like the one shown in the figure.

# 4.1.2 The Physical Layer

1. The correct answer is False. The physical layer provides the means to transport bits over the network whether the network is wired or wireless.
2. The correct answer is False. When encoded, the bits making up a frame are transmitted over the media one at a time.
3. The physical layer receives frames from the data-link layer and converts it to bits for transmission. On the sending device the physical layer passes the transmitted bits up to the data link layer as a complete frame.
4. The physical layer receives frames from the data link layer for encoding and transmission.

# 4.2.1 Physical Layer Standards

- The physical layer standards address three functional areas:

- Physical Components

- Encoding

- Signaling

# 4.2.2 Physical Components

The physical components are the electronic hardware devices, media, and other connectors that transmit the signals that represent the bits. Hardware components such as NICs, interfaces and connectors, cable materials, and cable designs are all specified in standards associated with the physical layer. The various ports and interfaces on a Cisco 1941 router are also examples of physical components with specific connectors and pinouts resulting from standards.

# 4.2.3 Encoding

- Encoding or line encoding is a method of converting a stream of data bits into a predefined "code”.

- Codes are groupings of bits used to provide a predictable pattern that can be recognized by both the sender and the receiver.

- Encoding is the method or pattern used to represent digital information. This is similar to how Morse code encodes a message using a series of dots and dashes.

- Manchester encoding represents a 0 bit by a high to low voltage transition, and a 1 bit is represented as a low to high voltage transition.

- Manchester encoding is used in older Ethernet standards such as 10BASE-T. Ethernet 100BASE-TX uses 4B/5B encoding and 1000BASE-T uses 8B/10B encoding.

- The transition occurs at the middle of each bit period.

# 4.2.4 Signaling

- The physical layer must generate the electrical, optical, or wireless signals that represent the "1" and "0" on the media. The way that bits are represented is called the signaling method.

- The physical layer standards must define what type of signal represents a "1" and what type of signal represents a "0". This can be as simple as a change in the level of an electrical signal or optical pulse. For example, a long pulse might represent a 1 whereas a short pulse might represent a 0.

# 4.2.5 Bandwidth

- Bandwidth is the capacity at which a medium can carry data. Digital bandwidth measures the amount of data that can flow from one place to another in a given amount of time.

- Bandwidth is typically measured in kilobits per second (kbps), megabits per second (Mbps), or gigabits per second (Gbps). Bandwidth is sometimes thought of as the speed that bits travel, however this is not accurate.

A combination of factors determines the practical bandwidth of a network:

- The properties of the physical media

- The technologies chosen for signaling and detecting network signals

- Physical media properties, current technologies, and the laws of physics all play a role in determining the available bandwidth.

# 4.2.6 Bandwidth Terminology

- Terms used to measure the quality of bandwidth include:

- Latency

- Throughput

- Goodput

- Latency: Latency refers to the amount of time, including delays, for data to travel from one given point to another.

- Throughput: Throughput is the measure of the transfer of bits across the media over a given period of time.

Goodput

There is a third measurement to assess the transfer of usable data; it is known as goodput. Goodput is the measure of usable data transferred over a given period of time. Goodput is throughput minus traffic overhead for establishing sessions, acknowledgments, encapsulation, and retransmitted bits. Goodput is always lower than throughput, which is generally lower than the bandwidth.

# 4.3.1 Characteristics of Copper Cabling

- Networks use copper media because it is inexpensive, easy to install, and has low resistance to electrical current. However, copper media is limited by distance and signal interference.

- Data is transmitted on copper cables as electrical pulses. A detector in the network interface of a destination device must receive a signal that can be successfully decoded to match the signal sent. However, the farther the signal travels, the more it deteriorates. This is referred to as signal attenuation. For this reason, all copper media must follow strict distance limitations as specified by the guiding standards.

- Electromagnetic interference (EMI) or radio frequency interference (RFI) - EMI and RFI signals can distort and corrupt the data signals being carried by copper media. Potential sources of EMI and RFI include radio waves and electromagnetic devices, such as fluorescent lights or electric motors.

- Crosstalk - Crosstalk is a disturbance caused by the electric or magnetic fields of a signal on one wire to the signal in an adjacent wire. In telephone circuits, crosstalk can result in hearing part of another voice conversation from an adjacent circuit. Specifically, when an electrical current flows through a wire, it creates a small, circular magnetic field around the wire, which can be picked up by an adjacent wire.

To counter the negative effects of EMI and RFI, some types of copper cables are wrapped in metallic shielding and require proper grounding connections.

To counter the negative effects of crosstalk, some types of copper cables have opposing circuit wire pairs twisted together, which effectively cancels the crosstalk.

The susceptibility of copper cables to electronic noise can also be limited using these recommendations:

- Selecting the cable type or category most suited to a given networking environment

- Designing a cable infrastructure to avoid known and potential sources of interference in the building structure

- Using cabling techniques that include the proper handling and termination of the cables

# 4.3.3 Unshielded twisted-pair (UTP)

- UTP cabling, terminated with RJ-45 connectors, is used for interconnecting network hosts with intermediary networking devices, such as switches and routers.

- In LANs, UTP cable consists of four pairs of color-coded wires that have been twisted together and then encased in a flexible plastic sheath that protects from minor physical damage. The twisting of wires helps protect against signal interference from other wires.

The numbers in the figure identify some key characteristics of unshielded twisted-pair cable:

- The outer jacket protects the copper wires from physical damage.

- Twisted-pairs protect the signal from interference.

- Color-coded plastic insulation electrically isolates wires from each other and identifies each pair.

# 4.3.4 Shielded twisted-pair (STP)

- Shielded twisted-pair (STP) provides better noise protection than UTP cabling. However, compared to UTP cable, STP cable is significantly more expensive and difficult to install. Like UTP cable, STP uses an RJ-45 connector.

- STP cables combine the techniques of shielding to counter EMI and RFI, and wire twisting to counter crosstalk. To gain the full benefit of the shielding, STP cables are terminated with special shielded STP data connectors. If the cable is improperly grounded, the shield may act as an antenna and pick up unwanted signals.

- The STP cable shown uses four pairs of wires, each wrapped in a foil shield, which are then wrapped in an overall metallic braid or foil.

The numbers in the figure identify some key features of shielded twisted-pair cable:

- Outer jacket

- Braided or foil shield

- Foil shields

- Twisted pairs

# 4.3.5 Coaxial cable

- Coaxial cable, or coax for short, gets its name from the fact that there are two conductors that share the same axis.

- A copper conductor is used to transmit the electronic signals.

- A layer of flexible plastic insulation surrounds a copper conductor.

- The insulating material is surrounded in a woven copper braid, or metallic foil, that acts as the second wire in the circuit and as a shield for the inner conductor. This second layer, or shield, also reduces the amount of outside electromagnetic interference.

- The entire cable is covered with a cable jacket to prevent minor physical damage.

The numbers in the figure identify some key features of coaxial cable:

- Outer jacket

- Braided copper shielding

- Plastic insulation

- Copper conductor

  - Coaxial cable, which is used for cable TV and internet service, is also used to attach antennas to wireless devices.
2. Shielded twisted-pair cable (STP) incorporates shielding and special connectors to prevent signal interference from other wires, EMI, and RFI.
3. Unshielded twisted-pair cable (UTP) is the most common type of wired network media.
4. Coaxial cable, which is used for cable TV and internet service and to attach antennas to wireless devices, uses several types of connectors to include BNC, N type, and F type connectors.

# 4.4 UTP Cabling

- When used as a networking medium, UTP cabling consists of four pairs of color-coded copper wires that have been twisted together and then encased in a flexible plastic sheath. Its small size can be advantageous during installation.

- Cancellation - Designers now pair wires in a circuit. When two wires in an electrical circuit are placed close together, their magnetic fields are the exact opposite of each other. Therefore, the two magnetic fields cancel each other and also cancel out any outside EMI and RFI signals.

- Varying the number of twists per wire pair - To further enhance the cancellation effect of paired circuit wires, designers vary the number of twists of each wire pair in a cable. UTP cable must follow precise specifications governing how many twists or braids are permitted per meter (3.28 feet) of cable. Notice in the figure that the orange/orange white pair is twisted less than the blue/blue white pair. Each colored pair is twisted a different number of times.

- UTP cable relies solely on the cancellation effect produced by the twisted wire pairs to limit signal degradation and effectively provide self-shielding for wire pairs within the network media.

The TIA/EIA-568 standard describes the wire color codes to pin assignments (pinouts) for Ethernet cables.

the RJ-45 connector is the male component, crimped at the end of the cable.

The socket, shown in the figure, is the female component of a network device, wall, cubicle partition outlet, or patch panel. When terminated improperly, each cable is a potential source of physical layer performance degradation.

This figure shows an example of a badly terminated UTP cable. This bad connector has wires that are exposed, untwisted, and not entirely covered by the sheath.

Different situations may require UTP cables to be wired according to different wiring conventions. This means that the individual wires in the cable have to be connected in different orders to different sets of pins in the RJ-45 connectors.

- Ethernet Straight-through - The most common type of networking cable. It is commonly used to interconnect a host to a switch and a switch to a router.

- Ethernet Crossover - A cable used to interconnect similar devices. For example, to connect a switch to a switch, a host to a host, or a router to a router. However, crossover cables are now considered legacy as NICs use medium-dependent interface crossover (auto-MDIX) to automatically detect the cable type and make the internal connection.

- Ethernet Straight-through - The most common type of networking cable. It is commonly used to interconnect a host to a switch and a switch to a router.

- Ethernet Crossover - A cable used to interconnect similar devices. For example, to connect a switch to a switch, a host to a host, or a router to a router. However, crossover cables are now considered legacy as NICs use medium-dependent interface crossover (auto-MDIX) to automatically detect the cable type and make the internal connection.

Note: Another type of cable is a rollover cable, which is Cisco proprietary. It is used to connect a workstation to a router or switch console port.

Using a crossover or straight-through cable incorrectly between devices may not damage the devices, but connectivity and communication between the devices will not take place. This is a common error and checking that the device connections are correct should be the first troubleshooting action if connectivity is not achieved.

The figure identifies the individual wire pairs for the T568A and T568B standards.

# 4.5 Fiber-Optic Cabling

- Single-mode fiber (SMF)

- Multimode fiber (MMF)

- One of the highlighted differences between MMF and SMF is the amount of dispersion. Dispersion refers to the spreading out of a light pulse over time. Increased dispersion means increased loss of signal strength. MMF has a greater dispersion than SMF. That is why MMF can only travel up to 500 meters before signal loss.

- Enterprise Networks - Used for backbone cabling applications and interconnecting infrastructure devices

- Fiber-to-the-Home (FTTH) - Used to provide always-on broadband services to homes and small businesses

- Long-Haul Networks - Used by service providers to connect countries and cities

- Submarine Cable Networks - Used to provide reliable high-speed, high-capacity solutions capable of surviving in harsh undersea environments at up to transoceanic distances. Search the internet for “submarine cables telegeography map” to view various maps online.

- 1. Multimode fiber has a shorter distance limitation than single-mode fiber. Commonly used on LANs with a distance of a few hundred meters but can be up to 2 km.
2. Multimode fiber used LEDs as the light source.
3. Single-mode fiber uses laser technology as the light source.
4. Single-mode fiber is commonly used for long haul TV and telephony applications.
5. Single-mode fiber is used for long haul applications up to 100 km.
6. Multimode fiber has a shorter distance limitation than single-mode fiber. Commonly used on LANs within a campus network.

# 4.6 Wireless Media

- Coverage area - Wireless data communication technologies work well in open environments. However, certain construction materials used in buildings and structures, and the local terrain, will limit the effective coverage.

- Interference - Wireless is susceptible to interference and can be disrupted by such common devices as household cordless phones, some types of fluorescent lights, microwave ovens, and other wireless communications.

- Security - Wireless communication coverage requires no access to a physical strand of media. Therefore, devices and users, not authorized for access to the network, can gain access to the transmission. Network security is a major component of wireless network administration.

- Shared medium - WLANs operate in half-duplex, which means only one device can send or receive at a time. The wireless medium is shared amongst all wireless users. Many users accessing the WLAN simultaneously results in reduced bandwidth for each user.

Although wireless is increasing in popularity for desktop connectivity, copper and fiber are the most popular physical layer media for deployment of intermediary network devices, such as routers and switches.

physical layer specifications are applied to areas that include the following:

- Data to radio signal encoding

- Frequency and power of transmission

- Signal reception and decoding requirements

- Antenna design and construction

  - The correct answer is False. Wireless provides the greatest mobility of all media and is gaining popularity in enterprise networks.
2. The correct answer is False. WLANs operate in half-duplex, which means only one device can send or receive at a time. This can impact network performance if there are many users accessing the WLAN at the same time.
3. Zigbee is intended for applications that require short-range, low data-rates, and long battery life, making it well suited for industrial and IoT applications.
4. This wireless standard is used for Personal Area Networks (PANs) and allows devices to communicate over distances of 1 to 100 meters.