To determine the destination MAC address of the frame when it comes out of R1, let's analyze the communication flow:

- PC1 wants to communicate with PC2, which is on a different network. This means PC1 will send the packet to its default gateway (R1).

- PC1 encapsulates the packet with:

  - Source MAC: PC1's MAC (aa-aa-aa)

  - Destination MAC: R1's MAC (bb-bb-bb)

- R1 receives the frame, de-encapsulates it, and checks the destination IP address. Since PC2 is on a different network, R1 forwards the packet to R2.

- When R1 forwards the frame to R2, it will use:

  - Source MAC: R1's outgoing interface MAC (cc-cc-cc)

  - Destination MAC: R2's MAC (dd-dd-dd)

- The correct answer is dd-dd-dd because it is the MAC address of R2, the next hop in the path to PC2.

Thus, the correct option to select is "dd-dd-dd".

4o

You said:

The first part of the MAC address is considered ______ Question 2 options: Organization Unique Identifier Old Unit Interface Organic University interconnect Option Unit Interface

ChatGPT said:

ChatGPT

The correct answer is:

Organization Unique Identifier (OUI)

The first 24 bits (or first half) of a MAC address represent the Organization Unique Identifier (OUI), which is assigned by the IEEE to manufacturers. This identifies the company that produced the network interface card (NIC). The remaining 24 bits are vendor-assigned.

4o

You said:

The data link layer has two sublayers. Question 3 options: Logical link control Media access control Application Transport

ChatGPT said:

ChatGPT

The correct answers are:

- Logical Link Control (LLC)

- Media Access Control (MAC)

These are the two sublayers of the Data Link Layer in the OSI model:

- LLC (Logical Link Control): Handles error detection, flow control, and managing communication between devices.

- MAC (Media Access Control): Controls how devices access and transmit data on the network, including addressing via MAC addresses.

The Application and Transport layers belong to different layers of the OSI model.

Which sublayer of the Data link is responsible for encapsulation and media access control? Question 4 options: Logical link control Media access control Application transport

ChatGPT said:

ChatGPT

The correct answer is:

✅ Media Access Control (MAC)

The Media Access Control (MAC) sublayer of the Data Link layer is responsible for encapsulation and media access control. It handles how devices access the physical transmission medium and how frames are transmitted and received over the network.

4o

You said:

According to the video (we watched) Which of the following numbers is considered a well-known port number? Question 5 options: 0 to 1023 1024-49151 49152-65535

ChatGPT said:

ChatGPT

The correct answer is:

✅ 0 to 1023

These numbers are known as well-known port numbers and are assigned by IANA (Internet Assigned Numbers Authority) for widely used protocols like HTTP (80), HTTPS (443), FTP (21), and SSH (22).

4o

You said:

CSMA/CD works on which layer of the OSI model? Question 6 options: Application Network Data link Physical

ChatGPT said:

ChatGPT

The correct answer is:

✅ Data Link Layer

Carrier Sense Multiple Access with Collision Detection (CSMA/CD) operates at the Data Link Layer (Layer 2) of the OSI model. It is used in Ethernet networks to detect and manage collisions when multiple devices attempt to transmit data simultaneously.

4o

You said:

Local hosts in a network have the same network portion of IP address as the default gateway. Question 7 options: True False

ChatGPT said:

ChatGPT

✅ True

Local hosts in a network share the same network portion of their IP address as the default gateway because they belong to the same subnet. The default gateway is typically the router that connects the local network to other networks, including the internet.

4o

You said:

According to this image, which of the following numbers are considered the Organization Unique Identifier (vendor specific)? Question 8 options: A1:B4:C5 C1:DD:3E DD:3E:A1 B4:C5:C1

ChatGPT said:

ChatGPT

✅ A1:B4:C5

The Organizationally Unique Identifier (OUI) is the first three octets (first 24 bits) of a MAC address. It is assigned by the IEEE to hardware manufacturers to identify their devices on a network.

Since the OUI consists of the first three hexadecimal pairs, A1:B4:C5 is the correct answer.

4o

You said:

Layer 2 addressing is used only for remote delivery. Question 9 options: True False

ChatGPT said:

ChatGPT

✅ False

Layer 2 addressing (MAC addresses) is used for both local and remote delivery.

- In a local network (LAN), Layer 2 addressing helps devices communicate directly within the same subnet using MAC addresses.

- For remote delivery, Layer 3 (IP addresses) is used to route packets across different networks, but Layer 2 is still involved in forwarding packets within each network segment.

4o

You said:

What destination MAC address would be included in a frame sent from a source device to a destination device on the same local network? Question 10 options: A broadcast MAC address of FF-FF-FF-FF-FF-FF The MAC address of the destination device The MAC address of the local router interface

ChatGPT said:

ChatGPT

✅ The MAC address of the destination device

When a source device sends a frame to another device on the same local network, it uses the MAC address of the destination device as the destination MAC address in the frame header.

- Broadcast MAC (FF-FF-FF-FF-FF-FF) is used when a device needs to send data to all devices in the network, like ARP requests.

- The MAC address of the local router interface is used only when sending data to a device on a different network (subnet).

4o

You said:

In lab 4, when you connected two routers, what type of cable did you use? (WAN to WAN) Question 11 options: Straight thru cable Crossover cable Fiber optic cable Coaxial cable

ChatGPT said:

ChatGPT

✅ Crossover cable

When connecting two routers directly via their WAN interfaces, a crossover cable is typically used.

Explanation:

- Crossover cables are used to connect similar devices (e.g., router-to-router, switch-to-switch, or PC-to-PC) because they swap the transmit and receive signals.

- Straight-through cables are used for different devices (e.g., PC-to-switch or router-to-switch).

- Fiber optic cables are used for long-distance, high-speed connections.

- Coaxial cables are used in cable internet connections but not for direct router-to-router links.

4o

You said:

In lab 3, what was the port number for a secure HTTP? Question 12 options: 423 413 443 433

ChatGPT said:

ChatGPT

✅ 443

The port number for secure HTTP (HTTPS) is 443. It is used for encrypted web traffic using SSL/TLS to ensure secure communication between clients and servers.

4o

You said:

In lab four, which logical port did you use to connect to the server Question 13 options: 9010 8088 1024 443

ChatGPT said:

ChatGPT

The correct answer depends on the specific setup in Lab 4, but based on common server configurations, the most likely answer is:

✅ 8088

Explanation:

- Port 9010 → Less common for web servers.

- Port 8088 → Often used as an alternative HTTP port for web servers.

- Port 1024 → The start of ephemeral ports, not typically used for server listening.

- Port 443 → Used for HTTPS (secure web traffic) but not always for custom server setups.

If your lab setup used a web server on a non-standard port, 8088 is a likely choice. Let me know if you need confirmation! 🚀

What is the command to create a static MAC address? Question 14 options: Arp -a Arp -s Arp -p Arp -d

To create a static MAC address, you would use the arp -s command. This allows you to manually add a static ARP entry.

If you have any other questions or need further help, feel free to ask!

Why do devices on the Ethernet network have to use two addresses (IP and MAC)? (select all that apply) Question 15 options: Because MAC addresses never change when you move your PC to another network Because IP addresses change when you move your PC to another network Because both MAC and IP addresses change when you move your PC to another network Because none of the MAC and IP addresses change when you move your PC to another network

Devices on the Ethernet network use both MAC (Media Access Control) and IP (Internet Protocol) addresses for several reasons:

- Because MAC addresses never change when you move your PC to another network: MAC addresses are hardware addresses that are unique to each network interface card (NIC) and do not change regardless of the network.

- Because IP addresses change when you move your PC to another network: IP addresses can change depending on the network you're connected to, as they are assigned by the network (e.g., via DHCP).

So the correct options are:

- Because MAC addresses never change when you move your PC to another network

- Because IP addresses change when you move your PC to another network

Need any more help or have other questions? Let's keep the conversation flowing!

According to the video, which of the following port numbers deals with email servers? Question 16 options: 21 80 53 25

ChatGPT said:

ChatGPT

Port 25 deals with email servers. It's used for SMTP (Simple Mail Transfer Protocol), which is responsible for sending emails between servers.

4o mini

You said:

The Data Link Address is used to forward the frame from one device to another within. Question 17 options: Different network segments The Internet The same device (inside the PC) The same network segment

ChatGPT said:

ChatGPT

The correct answer is The same network segment.

The Data Link Address (also known as the MAC address) is used to forward frames between devices within the same network segment. It helps in identifying devices on a local network.

4o mini

You said:

The limitations of wireless are: (select all that apply) Question 18 options: Coverage area Interference Security Shared medium

ChatGPT said:

ChatGPT

The limitations of wireless include:

- Coverage area

- Interference

- Security

- Shared medium

All of these factors can impact the performance, reliability, and security of wireless networks.

4o mini

You said:

A MAC address is _____ number Question 19 options: 48 bits 48 bytes 48 kilobytes 48 megabytes

ChatGPT said:

ChatGPT

A MAC address is a 48-bit number.

It is typically represented as a 12-digit hexadecimal number (6 bytes).

4o mini

You said:

At each hop along the path, a router performs __ basic Layer 2 functions: (select all that apply) Question 20 options: Accepts a frame from the network medium. De-encapsulates the frame to expose the encapsulated packet. Re-encapsulates the packet into a new frame. Forwards the new frame on the medium of the next network segment.

ChatGPT said:

ChatGPT

At each hop along the path, a router performs the following basic Layer 2 functions:

- Accepts a frame from the network medium.

- De-encapsulates the frame to expose the encapsulated packet.

- Re-encapsulates the packet into a new frame.

- Forwards the new frame on the medium of the next network segment.

These actions are part of how a router handles and forwards data across different network segments.

4o mini

You said:

What type of information is contained in an ARP table? Question 21 options: Routes to reach destination networks Domain name to IPv4 address mapping IPv4 address to MAC address mappings Switch ports associated with destination MAC address

ChatGPT said:

ChatGPT

The correct answer is:

IPv4 address to MAC address mappings.

An ARP (Address Resolution Protocol) table stores mappings of IPv4 addresses to their corresponding MAC addresses, which allows devices to communicate on a local network.

4o mini

You said:

Which of the OSI model layers adds a Trailer (CRC) to the signal? Question 22 options: Application Physical Network Data link

ChatGPT said:

ChatGPT

The correct answer is:

Data link

The Data Link layer (Layer 2) is responsible for adding a trailer, which typically contains a CRC (Cyclic Redundancy Check), to the frame to ensure error detection during transmission.

4o mini

You said:

Which media uses patterns of microwaves to represent bits? Question 23 options: copper Wireless fiber-optic RJ45

ChatGPT said:

ChatGPT

The correct answer is:

Wireless

Wireless media uses patterns of microwaves (or radio waves) to transmit data, where the information is encoded into these waves to represent bits.

4o mini

You said:

Wireless use ____________ standard Question 24 options: Wi-Fi (IEEE 802.1) Wi-Fi (IEEE 802.2) Wi-Fi (IEEE 802.19) Wi-Fi (IEEE 802.11)

ChatGPT said:

ChatGPT

The correct answer is:

Wi-Fi (IEEE 802.11)

The IEEE 802.11 standard defines the technology for wireless local area networks (WLANs), commonly known as Wi-Fi.

4o mini

You said:

Full-duplex communications with Ethernet switches do not require access control through CSMA/CD. Question 25 options: True False

ChatGPT said:

ChatGPT

The correct answer is:

True

In full-duplex communication, Ethernet switches can send and receive data simultaneously, which eliminates the need for access control through CSMA/CD (Carrier Sense Multiple Access with Collision Detection). CSMA/CD is primarily used in half-duplex communication to avoid collisions.

4o mini

You said:

Carrier sense multiple access with collision avoidance (CSMA/CA) is used on Wireless LAN. Question 26 options: True False

ChatGPT said:

ChatGPT

The correct answer is:

True

Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) is used in Wireless LANs (WLANs) to prevent collisions, as wireless communication doesn't have the same ability to detect collisions as wired networks. It helps manage access to the shared medium by avoiding potential conflicts before they happen.

4o mini

You said:

Name three types of copper cabling. Question 27 options: UTP STP Coaxial Fiber optic

ChatGPT said:

ChatGPT

The three types of copper cabling are:

- UTP (Unshielded Twisted Pair)

- STP (Shielded Twisted Pair)

- Coaxial

Fiber optic is not a type of copper cabling, as it uses light signals rather than electrical signals for data transmission.

4o mini

You said:

What PDU is received by the physical layer for encoding and transmission? Question 28 options: Frame Segment Packet Bit

ChatGPT said:

ChatGPT

The correct answer is:

Bit

At the physical layer, the PDU (Protocol Data Unit) is a bit. The physical layer is responsible for encoding and transmitting raw data (bits) over the physical medium.

4o mini

You said:

Which of the following commands shows the port number? Question 29 options: Arp -a Netstat -n ipconfig /all Cmd

ChatGPT said:

ChatGPT

The correct answer is:

Netstat -n

The netstat -n command shows active connections and listening ports along with their corresponding port numbers in numerical form.

When the destination IP address is on a remote network, the destination MAC address is that of the router that connects to the destination network.

In this case, the correct answer is "R1 MAC address". This is because when PC1 is sending a packet to a remote network (like the one PC2 is on), the packet's destination IP address is PC2's IP address. However, the destination MAC address is that of the next hop router, which is R1 in this scenario. The packet is sent to R1 first, which then forwards it to R2 and eventually to PC2. Hence, the correct answer is "R1 MAC address".

If you have any more questions or need further clarification, feel free to ask! 😊

Would you like to delve deeper into network concepts or perhaps explore something different?

why not R2?

Great question!

The reason why the destination MAC address is not R2's is because of the way routing works. When PC1 sends a packet to PC2, the packet needs to pass through the router R1 first. At this point in the journey, PC1 does not know about R2; it only knows that it needs to send the packet to R1 to reach the remote network.

Here's a simplified breakdown:

- PC1 sends the packet with PC2's IP address as the destination.

- However, the MAC address (layer 2) is R1's MAC address, as PC1 knows it needs to send the packet to R1 first.

- R1 receives the packet, checks its routing table, and forwards the packet to R2 (and so on until it reaches PC2). In this forwarding process, R1 changes the MAC address to that of R2.

This "next-hop" process ensures that the packet takes the correct path through the network. So at each stage, the packet's IP address remains the final destination's (PC2's), but the MAC address changes to that of the next hop router.

I hope that clarifies things! Do you have any other questions or anything else you'd like to explore? 😊

HOW DID YOU KNEW THAT PC1 IS SENDING THE INFO TO PC2?

I inferred that based on the context of your original question about the destination MAC address on a remote network. Typically, when discussing routing and networking scenarios, we talk about sending data from one device (like PC1) to another device (like PC2), often through one or more routers (like R1 and R2).

What is the destination MAC address if it is a broadcast frame? Question 32 options: 192.168.1.6 FF-FF-FF-FF-FF-FF 1-2-3-4-5-6 Ethernet doesn't have a broadcast frame.

ChatGPT said:

ChatGPT

The correct answer is:

FF-FF-FF-FF-FF-FF

This is the destination MAC address used for broadcast frames in Ethernet networks. Broadcast frames are intended for all devices on the network, so this special address is used to indicate that.

4o mini

You said:

When a NIC receives an Ethernet frame, it examines the ________ address to see if it matches the physical MAC address that is stored in RAM. Question 33 options: Source IP address Source MAC address Destination MAC address Destination IP address

ChatGPT said:

ChatGPT

The correct answer is:

Destination MAC address

When a NIC (Network Interface Card) receives an Ethernet frame, it examines the destination MAC address to see if it matches the physical MAC address stored in the NIC's RAM. If it matches, the frame is processed; if not, it is ignored.

4o mini

You said:

UTP wires are twisted to prevent the _________ Question 34 options: Attenuation Crosstalk Breakage cable None of the above

ChatGPT said:

ChatGPT

The correct answer is:

Crosstalk

UTP (Unshielded Twisted Pair) wires are twisted to prevent crosstalk, which is interference caused by signals from adjacent wires. The twisting helps minimize electromagnetic interference and maintain the quality of the signal.

4o mini

You said:

When the destination IP address is on a remote network, the destination MAC address will be the address of the host's ________ Question 35 options: Host's default gateway, the router's NIC Local default gateway, of the host's NIC Host's default gateway, the switch's NIC The host's IP address, the Hub's NIC

ChatGPT said:

ChatGPT

The correct answer is:

Host's default gateway, the router's NIC

When the destination IP address is on a remote network, the host sends the frame to its default gateway (typically a router), and the destination MAC address in the Ethernet frame will be the MAC address of the router's NIC. The router will then forward the packet towards its destination.

4o mini

You said:

A device uses ARP to determine the ________ address of a local device when it knows its IPv4 address. Question 36 options: Source IP address Destination MAC address Source MAC address Destination IP address

ChatGPT said:

ChatGPT

The correct answer is:

Destination MAC address

A device uses ARP (Address Resolution Protocol) to determine the destination MAC address of a local device when it knows its IPv4 address. ARP maps the IPv4 address to the corresponding MAC address.

4o mini

You said:

Which layer of the OSI model deals with bits cross? Question 37 options: Physical Network Transport Application

ChatGPT said:

ChatGPT

The correct answer is:

Physical

The Physical layer of the OSI model deals with the transmission of raw bits over a physical medium, such as electrical signals, light pulses, or radio waves. It is responsible for the actual hardware connection and the transmission of data in the form of bits.

4o mini

You said:

According to the video (we watched in class) what was the port number for FTP Question 38 options: 80 21 53 433

ChatGPT said:

ChatGPT

The correct answer is:

21

FTP (File Transfer Protocol) uses port 21 for communication. It is the standard port for FTP control connections.

4o mini

You said:

Layer 2 addresses are used to deliver packets from one NIC to another NIC on the same network. Question 39 options: True False

ChatGPT said:

ChatGPT

The correct answer is:

True

Layer 2 addresses, also known as MAC (Media Access Control) addresses, are used to deliver frames from one NIC (Network Interface Card) to another NIC on the same network. These addresses operate at the Data Link layer of the OSI model.

4o mini

You said:

To send a frame, a device will search its ARP table for a destination IPv4 address and a corresponding MAC address. Now, If there is no ARP table entry is found in the PC, then the device sends an Question 40 options: A unicast frame would be sent to the device directly A broadcast request frame would be sent to all devices A multicast request frame would be sent to all devices A multicast request frame would be sent to the device directly

ChatGPT said:

ChatGPT

The correct answer is:

A broadcast request frame would be sent to all devices

If there is no ARP table entry found for the destination IPv4 address, the device will send an ARP broadcast request frame to all devices on the local network to request the MAC address corresponding to the destination IP address. The device with the matching IP address will respond with its MAC address.

4o mini

You said:

UTP cable has ____ pairs of wires. Question 41 options: 4 6 8 10

ChatGPT said:

ChatGPT

The correct answer is:

4

A standard UTP (Unshielded Twisted Pair) cable has 4 pairs of twisted wires, making a total of 8 individual wires. These pairs are typically used for transmitting and receiving data in networks like Ethernet.

4o mini

You said:

What two protocols are used to determine the MAC address of a known destination device IP address (IPv4 and IPv6) (choose two) Question 42 options: DHCP ARP DNS ND

ChatGPT said:

ChatGPT

The correct answers are:

ARP (Address Resolution Protocol) for IPv4
ND (Neighbor Discovery) for IPv6

ARP is used to map an IPv4 address to a MAC address, while ND (Neighbor Discovery) performs a similar function for IPv6 addresses.

4o mini

You said:

Legacy Ethernet using a bus topology or hubs, is a shared, half-duplex medium Question 43 options: True False

ChatGPT said:

ChatGPT

The correct answer is:

True

Legacy Ethernet using a bus topology or hubs is indeed a shared, half-duplex medium. In this setup, all devices share the same communication channel, and data transmission can only occur in one direction at a time (half-duplex). Hubs also broadcast traffic to all connected devices, which can lead to collisions.

4o mini

You said:

In lab 4, what command did you use to obtain MAC addresses on your laptop? Question 44 options: Arp -a Arp -b Arp -c Arp -d

ChatGPT said:

ChatGPT

The correct answer is:

Arp -a

The arp -a command is used to display the ARP table, which includes the mapping of IP addresses to MAC addresses on a device. This command shows the MAC addresses of devices in the local network that your laptop has communicated with.

4o mini

You said:

The amount of time, including delays, for data to travel from one given point to another is called Question 45 options: Latency Goodput Throughput

ChatGPT said:

ChatGPT

The correct answer is:

Latency

Latency refers to the amount of time it takes for data to travel from one point to another, including any delays that may occur during transmission.

4o mini

You said:

Which of the following cables are immune to EMI? Question 46 options: Fiber optic UTP STT Coaxial

ChatGPT said:

ChatGPT

The correct answer is:

Fiber optic

Fiber optic cables are immune to electromagnetic interference (EMI) because they transmit data using light signals rather than electrical signals, making them unaffected by electrical noise or interference.

4o mini

You said:

The process that a source host uses to determine the destination MAC address associated with an IPv4 address is known as Question 47 options: Tracer ipconfig Arp ping

ChatGPT said:

ChatGPT

The correct answer is:

ARP (Address Resolution Protocol)

ARP is the process used by a source host to determine the destination MAC address associated with an IPv4 address. It sends a broadcast request to the network, and the device with the matching IP address replies with its MAC address.

4o mini

You said:

If the source MAC address does not exist in the MAC address table, then what's going to happen? Question 48 options: It is added to the table along with the incoming port number. it is deleted in the table along with the incoming port number. it is added to the router table along with the incoming port number. it is dropping the frame, along with the incoming port number.

ChatGPT said:

ChatGPT

The correct answer is:

It is added to the table along with the incoming port number.

If the source MAC address does not exist in the MAC address table of a switch, the switch will add the MAC address to the table along with the port from which the frame was received. This helps the switch forward future frames to the correct port.

4o mini

You said:

The port number in a PC is the physical number Question 49 options: True False

ChatGPT said:

ChatGPT

The correct answer is:

False

The port number in a PC refers to a logical number used by networking protocols (such as TCP or UDP) to identify specific processes or services running on the machine. It is not related to the physical hardware port. The physical ports on a PC are the actual connectors for devices, such as USB or Ethernet ports.

4o mini

You said:

Which of the following commands shows the ARP cache in your laptop? Question 50 options: Arp -d Arp -a Arp -s Arp -p

ChatGPT said:

ChatGPT

The correct answer is:

Arp -a

The arp -a command shows the ARP cache on your laptop, which includes the mappings of IP addresses to MAC addresses for devices on the local network.